Incident Investigation Toolkit

This Incident Investigation Toolkit contains a number of different resources for use by the investigator and/or the investigation team. .The toolkit includes data collection and analysis forms, checklists and summary guidance.  The various materials presented here can be downloaded in electronic form from the ABS website at “http://www.eagle.org/rules/downloads.html” under the publication entitled “ABS Guidance Notes on the Investigation of Marine Incidents”.

1	General Resource Materials

- Investigation Tools Checklist

- Simple Investigation Plan Form

- Detailed Investigation Plan Form

- Team Leader Responsibilities Checklist

- Meeting Notes Form

2	General Data Gathering Resource Materials

- Investigator’s Log Form

- Initial Call Checklist

- Data Needs Form

- Data Needs Checklist

- Initial Incident Scene Tour Checklist

- Post-Tour Checklist

- Open Issues Log Form

3	People Data Gathering Resource Materials

- Contacts Form

- Meetings Attendees Form

- Interview Scheduling Form

- Initial Witness Statement Form

- Interview Preparation Guidelines

- Interview Guidelines

- Interview Documentation Form

4	Data Control Resource Materials

- Data Log Form

- Data Correspondence Log Form

- Data Checkout Log Form

5	Physical Data Gathering Resource Materials

- Test Plan Form - Parts Analysis

- Test Plan Form – Sample/Chemical Analysis

6	Paper Data Gathering Resource Materials

- Paper Chart Data Collection Guidelines

7	Position Data Gathering Resource Materials

- Position Data Form

- Photography Guidelines – Stills

- Photography Guidelines – Video

- Photographic Record Form

8	Data Analysis Resource Materials

- 5-Whys Worksheet

- Root Cause Summary Table Form

- Causal Factor, Root Cause and Recommendation Checklist

- Recommendation Tracking Summary Form

- Management Resolution of Recommendations

- Change Analysis Worksheet

- HAZOP Worksheet

9	Report Resource Materials

- Incident Investigation Report Form

- Report Checklist

- Report Comment Form

- Report Routing Form

10	Investigation Resource Materials

- Investigation Checklist

- Incident Investigation/Root Cause Analysis Program Evaluation Checklist

11	Causal Factor Charts, 5-Whys and Fault Trees Resources

A simple Excel template for documenting causal factor charts and fault trees can be found on the ABS website at “http://www.eagle.org/rules/downloads.html” under the publication entitled “ABS Guidance Notes on the Investigation of Marine Incidents”.

Means for creating causal factors charts, 5-Whys and fault trees is provided in an Incident Investigation/Root Cause Analysis software tool available from ABS.

This Page Intentionally Left Blank

Investigation Tools Checklist

| Incident Number |  |  | Incident Date and Time |  | / / |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |  |  |  |
| Item No. | # Description |  |  |  |  |  | Quantity |  | Packed? |
| 1 | Forms (following forms with multiple copies**): |  |  | Number |  | Color | 1 Package |  |  |
|  | Detailed Investigation Checklist |  |  | 15 |  | White |  |  |  |
|  | Pocket Guide |  |  | 15 |  | Green |  |  |  |
|  | Data Log Form |  |  | 5 |  | Salmon |  |  |  |
|  | Interview Scheduling Form |  |  | 5 |  |  |  |  |  |
|  | Investigator’s Log |  |  | 5 |  | Gold |  |  |  |
|  | Initial Witness Statement Form |  |  | 15 |  | Blue |  |  |  |
|  | List of Contacts |  |  | 3 |  | Pink |  |  |  |
|  | Test Plan Form – Parts Analysis |  |  | 5 |  |  |  |  |  |
|  | Data Needs |  |  | 5 |  |  |  |  |  |
|  | Test Plan Form – Sample/Chemical Analysis |  |  | 5 |  |  |  |  |  |
|  | Avery 5160 label sheets |  |  | 4 |  |  |  |  |  |
|  | Photographic Record Form |  |  | 15 |  | Yellow |  |  |  |
|  | Meeting Notes Form: |  |  | 5 |  | Green |  |  |  |
| 2 | Root Cause Maps ___A3 (11 X 17) text only version: 5 copies ___Color version: 2 copies |  |  |  |  |  | 1 Package |  |  |
| 3 | ABS Guidance Notes on the Investigation of Marine Incidents |  |  |  |  |  | 1 |  |  |
| 4 | Electronic data storage means (e.g., floppy discs, blank CDs, thumb drive) |  |  |  |  |  | As required |  |  |
| 5 | Digital camera (in camera bag)* |  |  |  |  |  | 1 |  |  |
| 6 | ## Extra batteries for the digital camera (in camera bag)* |  |  |  |  |  | 8 AA |  |  |
| 7 | Serial cable to connect digital camera to laptop computer (in camera bag)* |  |  |  |  |  | 1 |  |  |
| 8 | Memory modules for digital camera (in camera bag)* |  |  |  |  |  | 2 |  |  |
| 9 | 128 mb Flash card for digital camera (in camera bag)* |  |  |  |  |  | 2 |  |  |
| 10 | Self-stick removable (Post-it) Notes 73 mm  73 mm (3”  3”), blue |  |  |  |  |  | 3 packages |  |  |
| 11 | Self-stick removable (Post-it) Notes 73 mm  73 mm (3”  3”), green |  |  |  |  |  | 6 packages |  |  |

Investigation Tools Checklist (continued)

| Item No. | # Description | Quantity | Packed? |
| --- | --- | --- | --- |
| 12 | Self-stick removable (Post-it) Notes 73 mm  73 mm (3”  3”), yellow | 3 packages |  |
| 13 | Self-stick removable (Post-it) Notes 73 mm  73 mm (3”  3”), pink | 2 packages |  |
| 14 | Self-stick removable (Post-it) Notes 73 mm  123 mm (3”  5”), yellow | 2 packages |  |
| 15 | Self-stick removable (Post-it) Notes 34.9 mm  47.6 mm (1½”  2”), yellow | 2 packages |  |
| 16 | Self-stick removable (Post-it) Notes 34.9 mm  47.6 mm (1½”  2”), blue | 2 packages |  |
| 17 | Self-stick removable (Post-it) Notes Arrows | 2 packages |  |
| 18 | Dry-erase pen (medium tip), blue | 2 |  |
| 19 | Dry-erase pen (wide tip), blue | 2 |  |
| 20 | Dry-erase pen (medium tip), black | 2 |  |
| 21 | Dry-erase pen (wide tip), black | 2 |  |
| 22 | Dry-erase pen (medium tip), red | 2 |  |
| 23 | Dry-erase pen (wide tip), red | 2 |  |
| 24 | Highlighter (wide tip), yellow | 2 |  |
| 25 | Highlighter (wide tip), green | 2 |  |
| 26 | Highlighter (wide tip), blue | 2 |  |
| 27 | Highlighter (wide tip), pink | 2 |  |
| 28 | Graph paper A4 (8½”  11”) | 2 pads |  |
| 29 | Lined paper A4 (8½”  11”) | 2 pads |  |
| 30 | Isometric graph paper | 1 pad |  |
| 31 | Binder clips, small | 1 box |  |
| 32 | Binder clips, medium | 1 box |  |
| 33 | Binder clips, large | 5 |  |
| 34 | Rubber bands | Various |  |
| 35 | Mechanical pencil | 2 |  |
| 36 | “Lead” suitable for mechanical pencil | 1 container |  |
| 37 | Large eraser | 1 |  |
| 38 | Ballpoint pen, blue | 2 |  |
| 39 | Ballpoint pen, red | 2 |  |
| 40 | Ballpoint pen, black | 3 |  |

Investigation Tools Checklist (continued)

| Item No. | # Description | Quantity | Packed? |
| --- | --- | --- | --- |
| 41 | Permanent felt-tipped pen (medium tip), blue | 2 |  |
| 42 | Permanent felt-tipped pen (medium tip), red | 2 |  |
| 43 | Permanent felt-tipped pen (medium tip), black | 2 |  |
| 44 | Blank adhesive labels 50 mm  90 mm (2”  3½”) for floppy disks | 20 |  |
| 45 | Flip chart paper, folded | 10 sheets |  |
| 46 | Assorted pieces of felt (for picture taking): white, beige, black | Large & Small |  |
| 47 | Small Scissors | 1 |  |
| 48 | Small mirror, folding | 1 |  |
| 49 | Small mirror (e.g., Boy Scout mirror) | 1 |  |
| 50 | Stapler with additional staples | 1 |  |
| 51 | Tape measure 30 m (100 ft)* | 1 |  |
| 52 | Retractable tape measure 7.5 m (25 ft)* | 1 |  |
| 53 | Straight-edged ruler 150 mm (6-inch), plastic | 1 |  |
| 54 | Straight-edged ruler 150 mm (6-inch), metal | 1 |  |
| 55 | Package of large latex gloves* | 2 packages |  |
| 56 | Heavy work gloves* | 2 pair |  |
| 57 | Clear plastic evidence bags, large – 1 liter (2.5 gallon) self-closing plastic bags* | 10 |  |
| 58 | Clear plastic evidence bags, medium – 0.5 liter (1 gallon) self-closing plastic bags* | 10 |  |
| 59 | Masking tape | 1 roll |  |
| 60 | Transparent tape | 1 roll |  |
| 61 | Multipurpose clipboard, plastic* | 1 |  |
| 62 | 75 mm (3-inch) blade pocket knife | 1 |  |
| 63 | Calipers (inside and outside measurements) | 1 |  |
| 64 | Waist pack, black | 1 |  |
| 65 | Flashlight (explosion proof), 3 cell with grommet for lanyard* | 2 |  |
| 66 | Batteries for flashlight (D size)* | 4 sets of 2 |  |
| 67 | Breakaway lanyards for clipboard and flashlight* | 3 |  |

Investigation Tools Checklist (continued)

| Item No. | # Description | Quantity | Packed? |
| --- | --- | --- | --- |
| 68 | Incident investigation data/evidence tags* | 20 (w/ties) |  |
| 69 | Investigator’s Business cards | 20 each |  |
| 70 | Software - ABS Incident Investigation Root Cause Analysis Software - Excel templates for causal factor charts and fault trees | 1 each 1 each |  |

*Items in field bag, all others in rolling bag

**Items in clipboards

Checklist completed by:   ________________________________   Date completed:   _______/_______/_______

Simple Investigation Plan Form

| Incident Number |  | Incident Date and Time |  |  | / / |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |  |
| Budget |  | Charge No. |  |  |  | Report Due |  |  | / / |
| Team Leader |  | Phone No. |  |  |  | Fax No. |  |  |  |
| Role |  |  | # Individual |  |  |  | Notes |  |  |
| Team leader |  |  |  |  |  |  |  |  |  |
| Investigation techniques |  |  |  |  |  |  |  |  |  |
| Navigation/Operations expertise |  |  |  |  |  |  |  |  |  |
| Machinery/Engineering expertise |  |  |  |  |  |  |  |  |  |
| Maintenance (mechanical, electrical, instrumentation and controls) |  |  |  |  |  |  |  |  |  |
| Structural engineering/support |  |  |  |  |  |  |  |  |  |
| Chartering issues/customer interface |  |  |  |  |  |  |  |  |  |

Simple Investigation Plan Form (continued)

| Role | # Individual | Notes |
| --- | --- | --- |
| Regulatory interface |  |  |
| Media interface |  |  |
| Restart/voyage resumption interface |  |  |
| Others/consultants |  |  |

Notes:

Date: _____/________/_______    		Page ___of___

Detailed Investigation Plan Form

| Incident Number |  | Incident Date and Time | / / |  |
| --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |

| ##### Legal Issues | Are legal issues a potential concern? If so, what kind (e.g., liability to public, liability to employees, insurance, regulatory, criminal)? |
| --- | --- |

| ##### Regulatory Impacts | Are there regulatory impacts? Is so, what kind (e.g., agencies involved and specific regulations)? |
| --- | --- |

| ##### Secure the incident scene | Work with emergency response personnel and incident response teams to stabilize the scene Notes: |
| --- | --- |

| ##### Vessel Status | What is the current status of the vessel? Total Loss? Fit to proceed? Unfit to proceed? What were the events leading up to this? |
| --- | --- |

| ##### Restart/ Resume Criteria | Restart or voyage resumption issues – what are the short-term and long-term concerns? |
| --- | --- |

| ##### Investigation Team | Select a team leader and team members based on the specifics of the incident. |
| --- | --- |

| Name | Role | Contact |
| --- | --- | --- |

Continued on next page

Detailed Investigation Plan Form (continued)

| ##### Supplies | Acquire Incident Investigation toolkit and other supplies |
| --- | --- |

| ##### Changes | Have any of the following occurred: |
| --- | --- |
|  | - Changes in operations – such as control systems, capacities, materials, locations, equipment, route |
|  | - Changes in systems that are related to the failure – support systems, auxiliary systems, bridge systems, machinery failures |
|  | - Changes in personnel – newly hired, newly transferred |
|  | - Changes in design, suppliers, maintenance practices |

| ##### Logisitics | Ensure that investigation team members have the required training and identification(safety briefing, personal protective equipment, respiratory protection, company identification etc.) to allow unescorted access to the investigation scene and team room |
| --- | --- |
|  | Identify a team room where appropriate: - secure so investigation materials can be left in the room - wall for causal factor chart and fault tree, etc. - flipchart paper, flipchart/easel, white board - phone/fax/copier |
|  | Determine locations for interviews separate from the team room, if needed away from the incident scene |
|  | Develop a list of team members (with titles and contact information) and their previous incident investigation training Note: Team members should be committed to the investigation on a full-time basis. If they are not, the investigation will be significantly impaired |
|  | Obtain overview of the operation, machinery, equipment etc involved. |
|  | Conduct a brief tour of the incident scene with escort if required. |
|  | Identify the need for any additional experts such as: – metallurgist, combustion issues, vendor representatives, marine chemists |

| ##### Data to Collect Immediately Logs | - Vessel/operations logs |
| --- | --- |
|  | Chief Engineers’s log |
|  | Computer logs for the last 24 hours (for printing if need be) |

Continued on next page

Detailed Investigation Plan Form (continued)

| ##### Data to Collect Immediately (continued) ##### People | List of potential witnesses (personnel should be available for interviews) |
| --- | --- |
|  | List of other personnel involved/related to the incident (personnel should be available for interviews) |
|  | List of personnel assigned to the vessel/operation/process etc. (personnel should be available for interviews) |
|  | List of emergency response personnel (personnel should be available for interviews) |

| ##### Vessel/ Machinery/ Operations | Log of operational and safety system alarms |
| --- | --- |
|  | Flow, temperature, pressure and other parameter trends |
|  | Operational sequence documentation |
|  | Navigational information (charts, radar records, communications data, etc.) |

| ##### Maintenance Status | Work permits and their status |
| --- | --- |
|  | Inspection reports and maintenance logs |

| ##### Materials | Composition reports |
| --- | --- |
|  | Analysis reports |
|  | Manifests |
|  | Welding procedures |
|  | Repair records |
|  | Product and intermediate specifications |

| ##### Photography | General photographs of the incident scene |
| --- | --- |
|  | Failed or damaged machinery/equipment/structures from multiple angles |
|  | Any indications of failure or damage |
|  | Stains, residues, foreign materials |

Continued on next page

Detailed Investigation Plan Form (continued)

| ##### Overview of Approach Introduction | Overview of the incident |
| --- | --- |
|  | Current status of the investigation |
|  | Current status of data gathering |
|  | Set up team room |

| ##### Initial tour | Complete safety briefings and other administrative requirements |
| --- | --- |
|  | See checklist for Initial Site Tour |

| ##### Data Analysis | Begin fault tree (or why tree) and/or causal factor charting. |
| --- | --- |
|  | Supplement with change analysis, if desired |
|  | Use root cause analysis map |

| ##### Interviews | Note: Interviews/data gathering will be mixed into analysis technique usage |
| --- | --- |
|  | Company/vessel/facility personnel should conduct the interviews (ask the questions) |
|  | One person does the interview while another takes notes but asks no questions |
|  | Interview guidelines be respectful be quiet no leading questions, no accusing questions tell me what you did, tell me what happened |

| ##### Test Plans | Physical parts analysis or sample/chemical analysis |
| --- | --- |
|  | Consider test plans for each item |

| ##### Reports | Begin report development from the beginning of the investigation |
| --- | --- |
|  | Schedule/process for completing the report |

Team Leader Responsibilities Checklist

| Incident Number |  | Incident Date and Time | / / |  |
| --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |

| ##### Direct and manage the team | - Obtains clear objectives for the investigation - Ensures that objectives of the investigation are accomplished - Ensures that the investigation is completed on schedule |
| --- | --- |

| ##### Control incident site access | Identify, control and if necessary, modify the restricted access zone. |
| --- | --- |

| ##### Safety | Ensure safety work practices are used at incident site during investigation.: |
| --- | --- |

| ##### Establish protocols | Establish administrative protocols for the investigation for: - Gathering data activities - Preserving data. |
| --- | --- |

| ##### Spokes person | Serve as the team’s spokesperson and point of contact for other groups and organizations |
| --- | --- |

| ##### Reporting | Keep others informed through status reports and other interim reports. - May make periodic verbal reports to management and staff, as required - Prepares interim written reports, as required |
| --- | --- |

| ##### Investigation Activities | Organizes team work for investigation activities - Assigns individuals to tasks and coordinates work with non-team members - Establishes schedules - Leads team meetings. |
| --- | --- |

Continued on next page

Team Leader Responsibilities Checklist (continued)

| ##### Integrity | Ensure team members maintain objectivity and commitment to the investigation |
| --- | --- |

| ##### Management of Resources | Obtains resources necessary for investigation - Processes required procurement documents or assigns a team member to this task - Initiates formal requests for: - Information, interviews, test results, technical or administrative support |
| --- | --- |

| ##### Controls impact | Minimizes the impact of the investigation on other activities |
| --- | --- |

| ##### Confidentiality | Protect proprietary and other sensitive information |
| --- | --- |

| ##### Final Report | Ensures that the final report if properly reviewed: - Factual accuracy of report for internal and external reports - Report is prepared for audience. - Review by legal department - Review by public relations department - Proprietary information protected. |
| --- | --- |

Meeting Notes Form

| ##### Details | Date | / / | Time |  |
| --- | --- | --- | --- | --- |
|  | Place |  |  |  |

| ##### Attendees |  |
| --- | --- |

| Name | Role/Company | Contact Info |
| --- | --- | --- |

| ##### Meeting purpose |  |
| --- | --- |

| ##### Topics discussed |  |
| --- | --- |

Continued on next page

Meeting Notes Form (continued)

| ##### Decisions made |  |
| --- | --- |

| Topic | Decision |
| --- | --- |

| ##### Actions |  |
| --- | --- |

| Topic | Who is Responsible | Deadline |
| --- | --- | --- |

| ##### Next Meeting |  |
| --- | --- |

Investigator’s Log Form

| Incident Number |  |  |  |  | Incident Date and Time | / / |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |
| Date | Start Time |  | End Time | Activity |  |  | Others Involved |  |

# Date: ____/____/________ 	Page____of____

Investigator’s Log Form (continued)

| Incident Number |  |  |  |  | Incident Date and Time | / / |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |
| Date | Start Time |  | End Time | Activity |  |  | Others Involved |  |

# Date: ____/____/________ 	Page____of____

Initial Call Checklist

| Incident Number |  |  | Incident Date and Time |  | / / |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |
| Company Name |  |  | Call Date and Time |  | / / |  |  |
| Name of Vessel/ Facility |  |  |  |  |  |  |  |
| Caller’s Name: |  |  |  |  |  |  |  |
| Caller’s Phone Number 1 |  |  |  | Caller’s Phone Number 2: |  |  |  |
| Caller’s Fax Number |  |  |  | Caller’s E-Mail Address |  |  |  |
| Item No. | Data Need |  | ## Response |  |  |  |  |
| 1 | Synopsis of incident |  |  |  |  |  |  |
| 2 | Current status of vessel/ installation - Equipment destroyed - Equipment shut down; awaiting restart - Reduced capacity - Normal operations - Schedule impacts |  |  |  |  |  |  |
| 3 | Current status of invest-igation - Emergency response? - Scene secured? - Initial witness statements obtained? - Initial investigation team commissioned? - Attorney/client privilege invoked? |  |  |  |  |  |  |

Initial Call Checklist (continued)

| Item No. | Data Need | ## Response |
| --- | --- | --- |
| 4 | Name/phone numbers of primary point of contact (POC) at vessel/installation (if different) |  |
| 5 | Name and phone numbers of other points-of-contact |  |
| 6 | Current location of vessel/ installation |  |
|  | Physical address, if appropriate |  |
| 7 | Two nearest commercial airports nearest to incident location |  |
| 8 | Recommended lodging arrangements (if needed) |  |
| 9 | Driving directions to port/ location from both of the nearest commercial airports |  |
| 10 | Expected arrival time at location for first meeting |  |
| 11 | Badging, security and safety procedures to enter location |  |
| 12 | Other information and notes |  |

Data Needs Form

| Incident Number |  |  |  |  | Incident Date and Time |  |  | / / |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |  |  |
| People |  |  | Position |  |  | Physical (components, chemicals) |  |  |  | Paper/Electronic (documentation, computer data) |  |
|  or X | Item |  |  or X | Item |  |  or X | Item |  |  |  or X | Item |
| • Keeping too much data is better than not keeping enough • Label all pieces of data and log them. This includes notes from interviews, procedures, computer disks, etc. |  |  |  |  |  | • Keep control of all evidence. Identify appropriate storage locations for all data • Use Chain of Custody forms for all data, even when no legal issues are anticipated |  |  |  |  |  |

Date: ____/____/________	Page____of____

Data Needs Checklist

| People | Position | Physical | Paper/Electronic |
| --- | --- | --- | --- |
| - Deck personnel - Engineering personnel - Steward department personnel - Operators - on-watch - off-watch - Personnel from other vessels - Personnel from other organizations such as - stevadores - dock workers - terminal operators - chandlers - agents - pilots - Maintenance personnel - Emergency responders - Warehouse personnel - Quality control personnel - Chartering representatives - Manufacturer’s representatives - Purchasing agents - Chemists - Metallurgists | - Document the position of: People – participants – observers – victims Physical – operating equipment – safety equipment – stains and residues – levels – instrument needles – chart recorder needles – switch positions – valve positions – relief devices – scattered objects – impact marks and scratches – burn/flame/scorch marks – layers of debris – environmental conditions (weather/seas) - Document and photograph what is on top of what in a pile of debris - Document and photograph the position of all equipment, switches, dials, etc. - Map and photograph all items before removal | - Document the physical aspects of: - Operating components - Safety devices - Support equipment - Structural components - Chemical samples – tanks – spills – raw materials – cargo - Retained samples - Stains - Residues - Foreign objects - Damaged equipment - Portable and temporary equipment - Instrumentation system components - Electrical switchgear - Security camera tapes - Use sealable plastic bags - Use durable tags to mark items that cannot be placed in a bag - Watch for incompatibility of sample containers and the samples | - Perform computer data capture as soon as possible - Document relevant information related to: - Procedures - Logs: bridge log, engine room log, cargo log, bell log - Computer records - Hazard and risk assessments - Policies and programs - Purchasing records - Design specifications and calculations - Training records/manuals - Management of change records - Maintenance and repair records - Previous incident reports - MSDS - Critical limits/setpoints - Software logic - Permits - Meteorological data - News media video - Vessel loading plans and records - Strip an wheel chart recorder plots - Work permits - Instrumentation loop and interlock drawings - Phone logs - Email printouts - Vessel general arrangement plans - Terminal plan - Charts - Depth recorder - Automatic radar plotting aid (ARPA) |

Initial Incident Scene Tour Checklist

| Incident Number |  |  | Incident Date and Time |  |  |
| --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |
| Investigator |  |  |  |  |  |
|  | Item | Step |  |  |  |
|  | 1 | The tour should be completed as soon as possible. |  |  |  |
|  | 2 | Obtain appropriate work permits - Hot work permits for photography |  |  |  |
|  | 3 | Follow all safe work practices identified by the incident scene commander or other safety personnel. |  |  |  |
|  | 4 | Don’t be in a rush to get to the center of the scene - Observe the big picture first - Walk through the entire area first. |  |  |  |
|  | 5 | Look not only at what is there, but what is NOT there |  |  |  |
|  | 6 | DO NOT MOVE ANYTHING unless absolutely necessary |  |  |  |
|  | 7 | Take notes - Positions of equipment/items/structures/outfitting - Distances, dimensions – measure and sketch or photograph - Orientations of equipment – measure and sketch or photograph - Scale, magnitude and extent of damage – note what is NOT damaged - Plan sample collection needs |  |  |  |
|  | 8 | Anticipate logistical challenges - Decide and arrange for long-term data/evidence storage - Plan for coordination with clean-up/remediation. |  |  |  |

Notes:

Date: ____/____/________

NOTE: If outside agencies are involved in investigating the incident, the initial site tour may need to be coordinated with these groups

Post-Tour Checklist

| Incident Number |  |  | Incident Date and Time | / / |  |
| --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |
| Investigator |  |  |  |  |  |
|  | Item | Step |  |  |  |
|  | 1 | Develop a detailed investigation plan - Review Simple or Detailed Investigation Plan (Incident Investigation Toolkit) - Develop detailed data needs (see Data Needs Form in Incident Investigation Toolkit). |  |  |  |
|  | 2 | Arrange for additional internal experts or personnel resources from other departments, organizations, vessels, as required - Legal assistance - Media interface assistance - Regulatory interface assistance. |  |  |  |
|  | 3 | Arrange for additional outside specialists or resources, as required, including: - Metallurgists - Structural engineers - Chemists - Explosion experts - Computer experts. |  |  |  |
|  | 4 | Establish data collection control procedures and assign personnel to: - Collect data - Inventory data - Control data. |  |  |  |

Notes:

Date: ____/____/________

Open Issues Log Form

| Incident Number |  |  | Incident Date and Time | / / |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |
| Issue Number | Issue |  |  |  | Assigned to | Date Assigned |  | Date Completed | Resolution |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |
|  |  |  |  |  |  | / / |  | / / |  |

Date: ____/____/________	Page____of____

Open Issues Log Form (continued)

| Incident Number |  |  | Incident Date and Time | / / |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |
| Issue Number | Issue |  |  |  | Assigned to |  | Date Assigned | Date Completed | Resolution |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |
|  |  |  |  |  |  |  | / / | / / |  |

Date: ____/____/________	Page____of____

Contacts Form

| Incident Number |  |  | Incident Date and Time |  | / / |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |
| Name |  | Title |  | Company |  | Phone/E-mail |  | Other |

Date: ____/____/________	Page____of____

Contacts Form (continued)

| Name | Title | Company | Phone/E-mail | Other |
| --- | --- | --- | --- | --- |

Date: ____/____/________	Page____of____

Meeting Attendees Form

| Incident Number |  |  | Incident Date and Time |  | / / |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |
| Meeting Date and Time | / / |  |  |  |  |  |  |  |
| Name |  | Title |  | Company |  | Phone/E-mail |  | Other |

Date: ____/____/________	Page____of____

Meeting Attendees Form (continued)

| Name | Title | Company | Phone/E-mail | Other |
| --- | --- | --- | --- | --- |

Date: ____/____/________	Page____of____

Interview Scheduling Form

| Incident Number |  |  | Incident Date and Time |  | / / |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |
| Interviewee (name and position) |  | Interviewer (name and position) |  | Expected Information |  | Priority |  | Scheduled | Completed |

Date: ____/____/________	Page____of____

Interview Scheduling Form (continued)

| Interviewee (name and position) | Interviewer (name and position) | Expected Information | Priority | Scheduled | Completed |
| --- | --- | --- | --- | --- | --- |

Date: ____/____/________	Page____of____

Initial Witness Statement Form

| Incident Date and Time | / / |  |  |
| --- | --- | --- | --- |
| Type of Incident | □ Accident or □ Near Miss |  |  |
| Your Name |  |  |  |
| Title |  | License or Certificate |  |
| Work Location |  |  |  |
| Work Telephone |  |  |  |

Please write your view of what happened as soon as possible after the incident.  Your cooperation in completing this form is appreciated and is an important part of the organization’s health, safety, environmental, reliability and quality programs.

Assignment of blame and fault finding are NOT part of the investigation process.  Punishment of individuals will ONLY occur in cases of illegal activity such as theft, use of illegal drugs, sabotage, etc.

Please provide information about what you know about incident:  Follow the WHO, WHAT, WHEN, WHERE, WHY thought process.

1.	Names of other people involved or in the area.

2.	Weather conditions.

3.	Anything moved or repositioned following the incident.

4.	Training and preparation issues.

Initial Witness Statement Form (continued)

5.	What happened? Please include each of the following:

- Timing of events (record sequentially and in as much detail as possible)

- Location of personnel

- Any indicators of the conditions that existed

- Actions of other people

- Emergency response activities

Return this document to the Incident Investigator.

Date: ____/____/________	Page___ of ____

Interview Preparation Guidelines

The following are typical questions that you may want to ask during an interview:

|  | Item | Question |
| --- | --- | --- |
|  | 1 | What was happening at or around the time of the incident? (i.e., initial conditions) |
|  | 2 | What were you doing just before the incident? |
|  | 3 | What were you doing during the incident? (i.e., timing of events) |
|  | 4 | What indications did you have of the incident? |
|  | 5 | How did you know what to do when you saw …? |
|  | 6 | What communications did you have with others in the area? |
|  | 7 | What other individuals were in the area? - where were they? - what were they doing? |
|  | 8 | What were the environmental conditions? |
|  | 9 | What was different this time? |
|  | 10 | Did you notice any equipment that didn’t operate properly? |
|  | 11 | Any training or preparation issues? |
|  | 12 | Emergency response: - What were the initial conditions when you arrived? - Did you or others move or reposition anything? - What emergency response activities did you perform? |
|  | 13 | Have there been similar events in the past? |
|  | 14 | Who else should we talk to? Who else might have information? |
|  | 15 | What are your opinions, beliefs and conclusions related to causes and recommendations |
|  | 16 | Is there anything else you wish to tell me? Is there anything else I should have asked? |

Notes:

Date: ____/____/________

Interview Guidelines

|  | Item | Guideline |
| --- | --- | --- |
|  | 1 | Use the Initial Witness Statement form to quickly capture people data - Review the Initial Witness Statements before the interview to help prepare for the interview - Don’t directly confront the witness with differences between the initial witness statement and statements made during the interview, but explore these differences |
|  | 2 | Use the Data Needs form and the Interview Scheduling form (if needed) |
|  | 3 | Keep witnesses separated. |
|  | 4 | Conduct interviews promptly. |
|  | 5 | Use data analysis techniques (fault trees, 5-Whys technique and causal factor charting) to develop a core set of questions. |
|  | 6 | Be respectful and be quiet. - Conduct interviews in neutral locations with as few distractions as possible - Interviews at the incident scene may also be appropriate - Perform interviews one-on-one or two-on-one - Never lead, accuse, blame or threaten the witness - Follow up on general comments to obtain clarifications and details |
|  | 7 | Document witness interviews |
|  | 8 | Review the notes from the interview with the witness |
|  | 9 | Assure confidentiality only if you can guarantee it. |

Notes:

Date: ____/____/________	Page___ of ____

Interview Documentation Form

| Interview Number |  | Interview Date and Time | / / |  |
| --- | --- | --- | --- | --- |
| Interviewer |  |  |  |  |
| Person being Interviewed |  |  |  |  |
| Others Present (Name/Company) |  |  |  |  |

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Page___ of ____

Interview Documentation Form (continued)

| Interview Number |  | Interview Date and Time | / / |  |
| --- | --- | --- | --- | --- |
| Person being Interviewed |  |  |  |  |

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

_________________________________________________________________________

Page___ of ____

Data Log Form

| Incident Number |  |  |  | Incident Date and Time |  | / / |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |
| ##### Data Type/Data Number |  |  |  |  |  |  | Description |  | Storage Location |
| Document | Electronic File |  | Component |  | Chemical |  |  |  |  |

Date: ____/____/________	Page____of____

Data Log Form (continued)

| ##### Data Type/Data Number |  |  |  | Description | Storage Location |
| --- | --- | --- | --- | --- | --- |
| Document | Electronic File | Component | Chemical |  |  |

Date: ____/____/________	Page____of____

Data Correspondence Log Form

| Incident Number |  |  | Incident Date and Time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |  |
| Document Number | Document Name |  |  | Sent to: |  |  |  | Sent by: | Date Sent: | Form Sent (paper, e-mail, floppy disk, CD, fax) |
|  |  |  |  | Group |  | Person |  |  |  |  |

Date: ____/____/________	Page____of____

Data Correspondence Log Form (continued)

| Document Number | Document Name | Sent to: |  | Sent by: | Date Sent: | Form Sent (paper, e-mail, floppy disk, CD, fax) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Group | Person |  |  |  |

Date: ____/____/________	Page____of____

Data Checkout Log Form

| Incident Number |  |  | Incident Date and Time |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |
| Item Number | ##### Description |  |  | Checked Out by |  | Checked Out | Returned | Initials |

Date: ____/____/________	Page____of____

Data Checkout Log Form (continued)

| Item Number | ##### Description | Checked Out by | Checked Out | Returned | Initials |
| --- | --- | --- | --- | --- | --- |

Date: ____/____/________	Page____of____

Test Plan Form — Parts Analysis

| Test Plan Number |  | Date and Time for Test | / / |  |
| --- | --- | --- | --- | --- |
| Test Plan for |  |  |  |  |
| Photos of Equipment |  |  |  |  |

Purpose of Test

Provide a brief description of the purpose of the test and the information that the test will provide.  Explain how this is related to the events and conditions on the causal factor chart, 5-Whys or fault tree.

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

Initial Preservation

| Persons performing data preservation |  | JSA Needed? |  yes  no |
| --- | --- | --- | --- |
| Location of Activity |  |  |  |
| Scheduled Date and Time | / / |  |  |

Visual Examination

| Persons performing visual examination |  | JSA Needed? |  yes  no |
| --- | --- | --- | --- |
| Location of Examination |  |  |  |
| Scheduled Date and Time | / / |  |  |

Operational Tests

| Persons performing operational tests |  | JSA Needed? |  yes  no |
| --- | --- | --- | --- |
| Location of Operational Test |  |  |  |
| Scheduled Date and Time | / / |  |  |

Test Plan Form — Parts Analysis (continued)

Field Disassembly

| Persons performing disassembly |  | JSA Needed? |  yes  no |
| --- | --- | --- | --- |
| Location of Disassembly |  |  |  |
| Scheduled Date and Time | / / |  |  |

Sampling (see Sample/Chemical Analysis Test Plan)

Shop/Bench Testing, Shop Disassembly

| Persons performing shop testing/ disassembly |  | JSA Needed? |  yes  no |
| --- | --- | --- | --- |
| Location of shop testing/disassembly |  |  |  |
| Scheduled Date and Time | / / |  |  |

Simulation

| Persons performing simulation |  | JSA Needed? |  yes  no |
| --- | --- | --- | --- |
| Location of simulation |  |  |  |
| Scheduled Date and Time | / / |  |  |

Testing (destructive and nondestructive)

| Persons performing testing |  | JSA Needed? |  yes  no |
| --- | --- | --- | --- |
| Location of simulation |  |  |  |
| Scheduled Date and Time | / / |  |  |

Test Plan Form — Parts Analysis (continued)

Long-term Preservation

| Persons performing long-term preservation |  | JSA Needed? |  yes  no |
| --- | --- | --- | --- |
| Location of Long-term preservation |  |  |  |
| Scheduled Date and Time | / / |  |  |

Approvals

| Test plan written by: |  | Title |  |
| --- | --- | --- | --- |
| Date and Time | / / |  |  |

| Test Plan Approved By | Title of Approver | Date Approved |
| --- | --- | --- |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |

Page __ of ___

This Page Intentionally Left Blank

Test Plan Form — Sample/Chemical Analysis

| Test Plan Number |  | Date and Time for Test | / / |  |
| --- | --- | --- | --- | --- |
| Test Plan for Sampling |  |  |  |  |
| Photos of Sampling Equipment/ Sample Point |  |  |  |  |

Purpose of Test

Provide a brief description of the purpose of the test and the information that the test will provide.  Explain how this is related to the events and conditions on the causal factor chart, 5-Whys or fault tree.

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

Drawing the Sample

| Persons drawing the sample |  | JSA Needed? | q yes q no |
| --- | --- | --- | --- |
| Location of sample equipment/point |  |  |  |
| Scheduled Date and Time | / / |  |  |

Note:  All equipment used in drawing the sample should be clean to prevent contamination of the sample.

Equipment needed for drawing sample (lines, hoses, containers, etc.):

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

Safety equipment (including personal protective equipment) required for drawing sample:

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

Description of sampling process (include container requirements, flushing times, volume of samples, etc.).  Attach JSA/procedure, if needed.

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

Test Plan Form — Sample/Chemical Analysis (continued)

Visual Examination of Sample Drawn/Collected

| Persons performing visual examination |  | JSA Needed? |  yes  no |
| --- | --- | --- | --- |
| Location of Visual Examination |  |  |  |
| Scheduled Date and Time | / / |  |  |

Marking of Containers

Mark the containers with the following information; also record the information below.

|  | Container Number |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Sample number |  |  |  |  |
| Container (size, material, color, etc.) |  |  |  |  |
| Time and date of sample | / / | / / | / / | / / |
| Description of what was sampled |  |  |  |  |

Transport/Storage of Samples

Persons _____________________________________________________________________

|  | Container Number |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Person involved |  |  |  |  |
| Storage location (if required) |  |  |  |  |
| Analysis to be performed on sample (refer to procedure number if appropriate) |  |  |  |  |
| Chain of custody form |  yes  no |  yes  no |  yes  no |  yes  no |
| *Analysis Report Number |  |  |  |  |

* Results (attach report to this form)

Test Plan Form — Sample/Chemical Analysis (continued)

Approvals

| Test plan written by: |  | Title |  |
| --- | --- | --- | --- |
| Date and Time | / / |  |  |

| Test Plan Approved By | Title of Approver | Date Approved |
| --- | --- | --- |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |
|  |  | / / |

Page __ of ___

This Page Intentionally Left Blank

Paper Chart Data Collection Guidelines

|  | Item | Guideline |
| --- | --- | --- |
|  | 1 | Identify all charts (strip charts, disk charts, etc.) that should be collected |
|  | 2 | Add all items to the list of paper data on the Data Log Form |
|  | 3 | DO NOT REMOVE THE PAPER FROM THE EQUIPMENT YET |
|  | 4 | Initial marking - Mark the name of the chart on the paper - Mark the parameter the chart is recording - Note: For multiple pen recorders, indicate the color and line associated with each parameter so that the association between the parameter and the trend data will still be possible with black and white copies - Mark the current time/date at the current location of the marker (to determine the speed of the recorder and provide a common reference across the various charts) |
|  | 5 | DO NOT REMOVE THE CHART YET |
|  | 6 | Wait a half-hour to an hour before proceeding to: - Mark the current time/date at the current location of the marker - The difference between the initial time/date mark and this second mark will allow you to determine the speed of the recorder |
|  | 7 | Remove the chart from the equipment and place in data storage |

Notes:

Date: ____/____/________	Page  ____ of ______

This Page Intentionally Left Blank

Position Data Form

| Incident Number |  |  | Incident Date and Time |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |
| Object Number | Description |  |  | Reference Point | Distance |  | Direction |  | Notes |

Date: ____/____/________	Page____of____

Position Data Form (continued)

| Object Number | Description | Reference Point | Distance | Direction | Notes |
| --- | --- | --- | --- | --- | --- |

Date: ____/____/________	Page____of____

Photography Guidelines – Stills

| Item | Guideline |
| --- | --- |
| 1 | Obtain hot work permits if necessary |
| 2 | Notify personnel in the area just before you take a photo using a flash. - Bright flashes of light are generally a cause for alarm. Warning personnel ahead of time will help address this issue. |
| 3 | Type of photos to use - Digital photography is acceptable for most investigations. - Digital photographs are usually not admissible in court proceedings. Standard film should be used in addition to any digital photos taken. - Instant photos can be used to assist the team in their investigation, but digital/film photos should also be taken of the same items. |
| 4 | Setting up the camera - Use automatic date and time stamping on each photograph if the camera has that capability. - Ensure that the date and time are properly set on the camera. - Use the highest resolution settings for the camera. - A camera with wide-angle and zoom capabilities is useful. |
| 5 | Setting up the Shot Plan the shot - Determine what you are trying to capture with the photo and plan the shot accordingly. Provide reference items in your photos - Use cardboard arrows, fluorescent tape, Post-it Notes, pens or people to point out and highlight items of interest in the photograph. - Provide an item of known dimension in the photograph – use a ruler (preferable) or other item of known dimension. - Provide reference points – an arrow pointing up, starboard or north in every photograph. Take photos from multiple angles - Begin with general views of the area. This will be helpful to put the more detailed photographs in context and show the relationship between each photo. - Include angles from witness locations to show what they would have seen during the incident. - Consider taking photographs at the same time of the day as the incident to reproduce the lighting conditions experienced by the witness. Use a non-reflecting background - Use of a cloth or felt background often helps to highlight the object in the photo and eliminates glare. - A selection of black, white and tan backgrounds usually works for most objects. - Before using the cloth/felt background, consider the potential contamination of the object from lint from the material. |

Photography Guidelines – Stills (continued)

| Item | Guideline |
| --- | --- |
| 6 | Document the photographs using the photographic log - Date and time of photo - Type of film used - Shutter speed (if known) - Key item of interest in the photograph (why it was taken) - Reference to drawing or document showing item - Direction of shot Examples Looking north from Hatch 4 Back side of control panel from boiler end Passageway 4-2 from aft entrance doorway - Distance from object of interest (if not readily identified by other objects in the photo). |
| 7 | Identity of photographer and recorder - Sign each documentation form - Initial each role of film. |
| 8 | Other considerations - Ensure you have extra sets of batteries available for the camera - Periodically checking of the batteries may be required to ensure the batteries are fresh. |

Photography Guidelines – Video

| Item | Guideline |
| --- | --- |
| 1 | Videotapes can be useful for: - seeing the relationship of one location to another - getting the big picture - capturing action, such as during the dismantlement of a component or during a simulation or test. |
| 2 | Do not count on videos to show details of components. |
| 3 | Start with an overview before zooming in on an object. |
| 4 | Do not move the camera too quickly – Pan/zoom/move twice as slowly as you think you need to. |
| 5 | Document the video using a voiceover on the tape to describe: - Date and time of video - Key item of interest in the photograph (why it was taken) - Direction of shot - Identity of photographer and recorder - Sign each documentation form - Initial each cassette - Provide reference items in your video. |
| 6 | Take videos from multiple angles. |

Photographic Record Form

| Incident Number |  |  | Incident Date and Time |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |  |  |
| Lighting |  |  |  | Film Roll |  |  |  |  |  |  |  |
| Number |  | Description |  |  | Time |  |  |  | Orientation |  | Paper Reference (documentation, drawing) |
|  |  |  |  |  | Date |  | Time |  |  |  |  |

Date: ____/____/________	Page____of____

Photographic Record Form (continued)

| Lighting |  |  | Film Roll |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number |  | Description |  | Time |  | Orientation |  | Paper Reference (documentation, drawing) |
|  |  |  |  | Date | Time |  |  |  |

Date: ____/____/________	Page____of____

5 Whys Worksheet

| Incident Number |  | Incident Date and Time | / / |  |
| --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |
| Investigator |  |  |  |  |
| Causal Factor Number and Description |  |  |  |  |
| Background: |  |  |  |  |
| Why: |  |  |  |  |
| Why: |  |  |  |  |
| Why: |  |  |  |  |
| Why: |  |  |  |  |
| Why: |  |  |  |  |
| Recommendations: |  |  |  |  |

Date: ____/____/________		Page __ of ___

5 Whys Worksheet (continued)

| Causal Factor Number and Description |  |
| --- | --- |
| Background: |  |
| Why: |  |
| Why: |  |
| Why: |  |
| Why: |  |
| Why: |  |
| Recommendations: |  |

Date: ____/____/________		Page __ of ___

Root Cause Summary Table Form

| Incident Number |  |  | Incident Date and Time | / / |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |
| Causal Factor # |  | Paths Through Root Cause Map™ |  |  | Recommendations |  |
| Description: |  |  |  |  |  |  |
| Background: |  |  |  |  |  |  |

Date: ____/____/________		Page __ of ___

Root Cause Summary Table Form (continued)

| Causal Factor # | Paths Through Root Cause Map™ | Recommendations |
| --- | --- | --- |
| Description: |  |  |
| Background: |  |  |

Date: ____/____/________		Page __ of ___

Causal Factor, Root Cause and Recommendation Checklist

| Incident Number |  |  | Incident Date and Time | / / |  |
| --- | --- | --- | --- | --- | --- |
| Incident Title |  |  |  |  |  |
| Investigator |  |  |  |  |  |
|  | Item | Guideline |  |  |  |
| Causal Factors |  |  |  |  |  |
|  | 1 | Oftentimes, it is a human error, usually committed by front-line personnel, though the causal factor could also be a structure, machinery/equipment, outfitting or external problem. |  |  |  |
|  | 2 | It is something we want to prevent from occurring in the future |  |  |  |
|  | 3 | Elimination or correction of the item will prevent the incident or reduce the consequences |  |  |  |
|  | 4 | Item is NOT a root cause |  |  |  |
| Date this section was completed: |  |  |  |  |  |
| Root Causes |  |  |  |  |  |
|  | 1 | A management system weakness |  |  |  |
|  | 2 | Addresses something over which management has control |  |  |  |
|  | 3 | Represents as deep a level of cause as is practical to correct through recommendations |  |  |  |
|  | 4 | Directly tied to a causal factor |  |  |  |
| Date this section was completed: |  |  |  |  |  |
| Recommendations |  |  |  |  |  |
|  | 1 | Directly tied to a root cause |  |  |  |
|  | 2 | Addresses options for reducing frequency and/or reducing the consequences of one or more root causes |  |  |  |
|  | 3 | Intended action clearly stated |  |  |  |
|  | 4 | Describes specific actions to be taken by management |  |  |  |
|  | 5 | Completion of the recommendation can be determined by reviewing data |  |  |  |
|  | 6 | Practical, feasible and achievable |  |  |  |
|  | 7 | Does not pose other undesirable and/or unforeseen risks |  |  |  |
|  |  | Short-term, medium-term and long-term recommendations addressed for each causal factor |  |  |  |
|  |  | Recommendations made at the highest level possible - Eliminate the possibility of recurrence - eliminate the hazard - Reduce the probability of occurrence - make the system inherently safer/more reliable or prevent the occurrence of the event - Reduce the consequences of the event - detect and mitigate the loss, contain the damage or perform emergency response |  |  |  |
|  |  | Four levels of recommendations considered for each root cause - Address the causal factor - Address the specific problem - Fix similar problems - Correct the process that creates these problems |  |  |  |
|  |  | Each recommendation has an assigned responsibility and a date for completion |  |  |  |
| Date this section was completed: |  |  |  |  |  |

Recommendation Tracking Summary

Investigation for:

Investigation/report #:

| Recommendation Number | Action | Responsible Person | Target Completion Date | Status |
| --- | --- | --- | --- | --- |

Date: ____/____/________	Page____of____

Management Resolution of Recommendations

Incident #: 		Date

Recommendation #(s):

Item(s):

|  | Recommendation Resolution | Person Responsible for Implementation |
| --- | --- | --- |
|  | Approved as recommended |  |
|  | Approved with modifications as documented 1,2,3 |  |
|  | Returned to investigation team for additional information 1 |  |
|  | Rejected because implementation of the recommendation would increase the overall risk of facility operations 1,3 |  |
|  | Rejected because the recommendation is no longer valid 1,3 |  |
|  | Rejected because implementation of other team recommendations adequately addresses this recommendation 1,3 |  |
|  | Rejected because the risk reduction associated with this item can be accomplished by a more effective (less costly, less complicated or greater risk reduction) action 1,2,3 |  |
|  | Rejected because the recommendation is not necessary to protect the health and safety of personnel 1,3 |  |
|  | Rejected because the recommendation is infeasible 1,2,3 |  |

1Explanation:

2Alternative action:

3Decision communicated to investigation team:

Prepared by:

Approved by:

This Page Intentionally Left Blank

Change Analysis Worksheet

| Incident Number |  |  | Incident Date and Time |  |  | / / |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Problem Situation (Describe) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Problem-free Situation (Describe) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check mark type of analysis being performed: |  | □ Actual |  | □ Test | □ Procedure |  |  | □ Ideal |  | □ Experience |  | □ Future | Other Specify |
| Conditions Found in Problem Situation |  | ##### Conditions Found in Problem-free Situation |  |  |  |  | Differences Between the Two Situations |  |  |  | Resulting Effects on Problem |  |  |

Date: ____/____/________	Page____of____

Change Analysis Worksheet (continued)

| Check mark type of analysis being performed: | □ Actual | □ Test | □ Procedure |  | □ Ideal | □ Experience |  | □ Future | Other Specify |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conditions Found in Problem Situation | ##### Conditions Found in Problem-free Situation |  |  | Differences Between the Two Situations |  |  | Resulting Effects on Problem |  |  |

Date: ____/____/________	Page____of____

HAZOP Worksheet

| Incident Number |  |  | Incident Date and Time |  | / / |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |
| Procedure |  |  |  |  | Intent |  |  |  |  |
| Step |  |  |  |  | References |  |  |  |  |
| Item Number | Deviation |  | Causes | Consequences |  |  |  | Safeguards | Recommendations |

Date: ____/____/________	Page____of____

HAZOP Worksheet (continued)

| Item Number | Deviation | Causes | Consequences | Safeguards | Recommendations |
| --- | --- | --- | --- | --- | --- |

Date: ____/____/________	Page____of____

Incident Investigation Report Form

| General |  |
| --- | --- |
| Vessel Name |  |
| Incident Type |  Near Miss or  Accident |
| Official No. |  |
| IMO No. |  |
| Incident Title |  |
| Initial Event (Grounding, Collision, etc) |  |
| Incident Date | / / |
| Incident Time |  |
| Report Date | / / |
| Report Number |  |
| Reported By |  |
| Class Affected? |  Yes or  No |
| Third Party Involved? |  Yes or  No |

| # Vessel Info |  |
| --- | --- |
| Call Sign |  |
| Flag Name |  |
| Port Of Registry |  |
| Classification Society |  |
| Registered Owner |  |
| Managing Company |  |
| Contact |  |
| Address |  |
| Tel No. |  |
| Mobile No. |  |
| E-Mail |  |
| Other |  |
| Vessel Type |  |
| Vessel Function |  |
| Length Overall |  |
| Gross Tonnage (ITC) |  |
| Net Tonnage (ITC) |  |
| Design Deadweight |  |
| Prime Mover |  |
| Maneuvering System |  |
| Propulsion System |  |
| Hull Material |  |
| Hull Construction |  |
| Delivery Date |  |
| Major Conversion Date | / / |
| Builder |  |

Incident Investigation Report Form (continued)

| Preliminary Data |  |
| --- | --- |
| Damage Details: |  |
| Emergency Response Measures: |  |
| Item-Failure Type and Failure Description |  |

| ## Consequence Summary |  |
| --- | --- |
| Damage to Vessel |  |
|  |  Fit to proceed  Unfit to Proceed  Total Loss |
| Damage to People |  |
| No. of Dead/Missing Crew: |  |
| No. of Dead/Missing Passengers: |  |
| No. of other Dead/Missing People: |  |
| No. of Seriously Injured Crew: |  |
| No. of Seriously Injured Passengers: |  |
| No. of other Seriously Injured People: |  |
| Damage to Environment |  |
| Bunker Spills (specify type and units*) |  |
| - Heavy fuel |  |
| - Diesel |  |
| - Lube oils |  |
| - Other |  |
| - Oil Cargo Spills |  |
| - Crude Oil |  |
| - Persistent Refined Oil Products |  |
| - Non-Persistent Refined Oil Products |  |
| *Specify Units next to each entry or pick one below to apply to all  Liters,  Barrels,  US Gallons,  Imperial Gallons,  Other (specify): |  |
| Chemicals In Bulk Spills |  kilos  lbs  Other: ______ |
| Lost Dangerous Goods In Packaged Form |  kilos  lbs  Other: ______ |

Incident Investigation Report Form (continued)

| People |  |
| --- | --- |
| No. of Crew |  |
| No. of Passengers |  |
| Pilot Onboard |  Yes or  No |
| People Involved In Incident |  |
| Name |  |
| Company |  |
| Role*: |  |
| Status (Dead, Missing, Injured, Uninjured) |  |
| Statement |  Yes - Append statements to the end of the report  No |
| Name |  |
| Company |  |
| Role*: |  |
| Status (Dead, Missing, Injured, Uninjured) |  |
| Statement |  Yes - Append statements to the end of the report  No |
| Name |  |
| Company |  |
| Role*: |  |
| Status (Dead, Missing, Injured, Uninjured) |  |
| Statement |  Yes - Append statements to the end of the report  No |
| Name |  |
| Company |  |
| Role*: |  |
| Status (Dead, Missing, Injured, Uninjured) |  |
| Statement |  Yes - Append statements to the end of the report  No |
| Name |  |
| Company |  |
| Role* |  |
| Status (Dead, Missing, Injured, Uninjured) |  |
| Statement |  Yes - Append statements to the end of the report  No |
| Name |  |
| Company |  |
| Role* |  |
| Status (Dead, Missing, Injured, Uninjured) |  |
| Statement |  Yes - Append statements to the end of the report  No |
| Name |  |
| Company |  |
| Role* |  |
| Status (Dead, Missing, Injured, Uninjured) |  |
| Statement |  Yes - Append statements to the end of the report  No |

* Choose one of following for Role: (Person(s) in Charge, Engineer of the Watch, Officer of the Watch, Crew, Witness Only, Pilot, Investigation Team, Passenger, Other)

Incident Investigation Report Form (continued)

| #### People Details |  |
| --- | --- |
| Name: |  |
| Role* | [Field Contents] |
| Status with regard to vessel: |  Crew member  On Duty  Off Duty  Not a crew member  Not applicable |
| Company: |  |
| Job Title |  |
| License/Certificate: |  |
| Date of Birth: |  |
| Nationality |  |
| Contact Details |  |
| Address |  |
| E-mail: |  |
| Tel No. |  |
| Mobile No.: |  |
| Other |  |
| Injury Details |  |
| Type of Injury: |  |
| Body Part Involved: |  |
| Heath Condition: |  |
| Where injury occurred: |  |
| Equipment/Substance Involved: |  |
| No. of Hrs worked before Incident: |  |
| Duration of last off duty period |  |

* Choose one of following for Role: (Person(s) in Charge, Engineer of the Watch, Officer of the Watch, Crew, Witness Only, Pilot, Investigation Team, Passenger, Other)

Incident Investigation Report Form (continued)

| Statement Details |  |
| --- | --- |
| Date of Statement |  |
| Place Statement Given |  |
| Taken by: |  |
| Statement: |  |
| Remarks |  |

Incident Investigation Report Form (continued)

| #### Voyage Conditions |  |
| --- | --- |
| Location |  |
| In Port |  |
| - Port |  |
| - Country |  |
| At Sea |  |
| - Latitude |  |
| - Longitude |  |
| - Place |  |
| - Departure Date: | / / |
| - Departure Time: |  |
| - To: |  |
| - From: |  |
| Operations |  |
| - Operational Status |  |
| - Voyage Phase: |  |
| - Underway Course True |  |
| - Underway Speed Knots |  |
| - As Loaded Draft (fwd): |  m or  ft |
| - As Loaded Draft (aft): |  m or  ft |
| Visibility |  |
|  |  Good  Fair  Poor |
| - Nautical Miles |  |
| - Ambient Outdoor Light |  |
| Atmospheric/Sea Conditions |  |
| - Sea State: |  |
| - Wind Speed: |  |
| - Wind Direction: |  |
| - Bottom Depth Under Keel: |  m or  ft |
| - Currents/Tides: |  |
| - Ice Conditions: |  |
| - Other: |  |

Incident Investigation Report Form (continued)

| #### Analysis |  |  |
| --- | --- | --- |
| Event description/Sequence |  |  |
| Root Cause Analysis Map paths |  |  |
| Principal findings |  |  |
| Attachments (This includes Causal factor charts, Fault or 5-Whys Trees) |  | Yes or No |

| #### Continuous Improvement |  |
| --- | --- |
| Investigate further? | Yes or  No |
| Recommendations to prevent recurrences |  |

Incident Investigation Report Form (continued)

#### Notes

Notes:

Report Checklist

| Incident Number |  |  | Incident Date and Time | / / |  |
| --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |
| Investigator |  |  |  |  |  |
|  | Item | Guideline |  |  |  |
| Interim Reports |  |  |  |  |  |
|  | 1 | All pages marked DRAFT – PRELIMINARY – BUSINESS CONFIDENTIAL or other appropriate markings |  |  |  |
|  | 2 | Report date, version number and pages marked “Page x of y” |  |  |  |
|  | 3 | Line numbering used to help with comment resolution |  |  |  |
|  | 4 | Controlled distribution of all copies with each copy marked with copy number |  |  |  |
|  | 5 | Cover page indicates that facts, causes, conclusions and recommendations are preliminary and may change |  |  |  |
|  | 6 | All copies returned after review |  |  |  |
| Final Reports |  |  |  |  |  |
|  | 1 | All pages marked BUSINESS CONFIDENTIAL or other appropriate markings |  |  |  |
|  | 2 | Report date, version number and pages marked “Page x of y” |  |  |  |
|  | 3 | Completed approval form attached |  |  |  |
| All Reports |  |  |  |  |  |
|  | 1 | Executive summary including a summary of the event, consequences (actual and potential), causal factors, observations, root causes and recommendations Note: completing a standard report form meets this requirement |  |  |  |
|  | 2 | Causal factor chart or fault tree for more complex events. 5-Whys for simpler events |  |  |  |
|  | 3 | Description of the incident – sufficient for the target reader to understand the incident – reference to a causal factor chart or fault tree can significantly reduce the text description Initial conditions, personnel involved (by position only, no names), consequences of the incident and significance of the incident to the stakeholders |  |  |  |
|  | 4 | Causal factors - Human errors and equipment failures - Failures to prevent the incident and failure to mitigate the consequences |  |  |  |
|  | 5 | Successful safeguards that significantly impacted the consequences identified. |  |  |  |
|  | 6 | No names used in the report – use sufficient identification of individuals to understand incident |  |  |  |
|  | 7 | Suppositions, opinions and conclusions clearly identified |  |  |  |

This Page Intentionally Left Blank

Report Comment Form

| Incident Number |  |  |  |  |  | Incident Date and Time |  |  | / / |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |  |  |  |  |  |  |
| Investigator |  |  |  |  |  |  |  |  |  |  |  |
| Report Date |  | / / |  |  |  | Report Version |  |  |  |  |  |
| Reviewer’s Initials | Section |  | Page Number |  | Line Number |  |  | ### Comment (use multiple lines if needed) |  |  | Resolution |
|  |  |  | Start | End | Start |  | End |  |  |  |  |

## Page __ of __

Report Comment Form (continued)

| Reviewer’s Initials | Section | Page Number |  | Line Number |  | ### Comment (use multiple lines if needed) | Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Start | End | Start | End |  |  |

## Page __ of __

Report Routing Form

I acknowledge that the findings and corrective actions related to Incident Report #: 	
have been reviewed with me.

| Employee/Contractor |  |  |  |
| --- | --- | --- | --- |
| Name (please print) | Signature | Position | Date |

Return completed form (i.e., signed) to the Safety Manager.

Date: ____/____/________	Page___ of ____

Investigation Checklist

| Incident Number |  |  | Incident Date and Time | / / |  |
| --- | --- | --- | --- | --- | --- |
| Incident Title: |  |  |  |  |  |
| Investigator |  |  |  |  |  |
|  | Item |  |  |  |  |
|  | All items on Data Needs form addressed (collected or decision made not to collect) |  |  |  |  |
|  | All items on the Open Issues Log addressed (resolved or decision made not to resolve) |  |  |  |  |
|  | Fault tree, 5-Whys and/or Causal factor chart complete - All questions answered or decision made not to resolve - All causal factors identified |  |  |  |  |
|  | Root causes identified for all causal factors |  |  |  |  |
|  | Recommendations developed for all root causes |  |  |  |  |
|  | Responsibilities and completion dates assigned for all recommendations |  |  |  |  |
|  | Report written and reviewed by all appropriate personnel |  |  |  |  |
|  | Report findings distributed at appropriate level of detail to all those involved in the event and in follow-up activities |  |  |  |  |
|  | Stakeholder meeting conducted |  |  |  |  |

# Completed by:  __________________________________________

# Date:  ____/____/______

Incident Investigation/Root Cause Analysis Program Evaluation Checklist

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 1 |  | General |  |
| 1.1 | RR | Is a written procedure in place to define the incident investigation process? |  |
| 1.2 | RR | Does the program identify the organization’s objectives for performing incident investigation? |  |
| 1.3 | RR | Does the organization have specific definitions of the following: - Causal factor (human errors and equipment failures) - Root cause - Recommendation - Item of note or observation |  |
| 1.4 | RR | Are the definitions for causal factor, root cause, recommendation and item of note or observation used consistently in the application and use of the program? |  |
| 1.5 | I-PTM I-SRQM | What are the definitions of causal factor, root cause and item-of-note (or observation)? |  |
| 1.6 | I-PTM | What are the team member’s responsibilities outlined in the investigation program? |  |
| 1.7 | I-SRQM | What are the safety/quality/reliability manager’s responsibilities outlined in the investigation program? |  |
| 2.1 |  | Incident Identification |  |
| 2.1.1 | RR | Does the incident investigation program specifically define the types of incidents that should be analyzed? Are examples of each type of incident included in the program? |  |
| 2.1.2 | RR | Do investigation procedures define the various levels of investigations (i.e., major loss, minor loss, near miss)? |  |
| 2.1.3 | RR | Do the investigation procedures describe the process and level of effort associated with each level of analysis? |  |
| 2.1.4 | RR | Does the investigation program specifically assign the task of incident reporting to someone? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 2.1.5 | I-SRQM | Are there documented thresholds for reporting incidents? |  |
| 2.1.6 | I-PTM I-GM I-SRQM | How is an incident investigation initiated (i.e., who is called, when, which type of form is used)? |  |
| 2.1.7 | I-GE | How do you report an incident? |  |
| 2.2 |  | Initiating the Investigation |  |
| 2.2.1 | RR I-ERM | Are emergency response activities performed with no interference from the II/RCA team? |  |
| 2.2.2 | RR I-PTM | Is an incident classification methodology used to allocate resources to investigations? |  |
| 2.2.3 | I-SRQM | Are there documented thresholds for reporting incidents? |  |
| 2.2.4 | RR | Is the classification methodology based on a measure of the ability to learn from the incident? |  |
| 2.2.5 | RR I-SRQM | Is the classification scheme is used? |  |
| 2.2.6 | RR | For acute events, have incident investigations been initiated as promptly as possible and always within 48 hours? |  |
| 2.2.7 | I-PTM I-SRQM I-GM | How soon must an investigation be conducted following a report of an incident? |  |
| 2.2.8 | RR I-PTM | Was the subject and scope of each investigation clearly defined by the commissioning body? |  |
| 2.2.9 | RR | Have incident investigation teams been established in accordance with the written incident investigation program? |  |
| 2.2.10 | RR | Do the teams contain at least one person knowledgeable in the process/system/equipment involved in the loss and other members with appropriate knowledge and experience to thoroughly investigate and analyze the events? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 2.2.11 | RR | Does each team contain personnel that are knowledgeable in the actual performance of the task and the desired performance of the task? |  |
| 2.2.12 | RR | Is the makeup of each investigation team consistent with the classification guidance? |  |
| 2.2.13 | I-PTM I-GM I-SRQM | How are incident investigation teams selected? Do the teams contain at least one person knowledgeable in the process and other persons with appropriate knowledge and experience to thoroughly investigate the incident? |  |
| 2.2.14 | RR I-PTM | Are adequate resources and skills made available to the incident investigation teams, depending on the complexity or seriousness of the incidents? |  |
| 2.2.15 | I-PTM I-SRQM | Do team members need any special skills and/or knowledge? |  |
| 2.2.16 | I-PTM I-SRQM | Is any type of training provided to employees in incident investigation skills and techniques? |  |
| 2.2.17 | RR I-SRQM | Do procedures allow for third-party participation in investigations and identify under what circumstances third parties might be required? |  |
| 3 |  | Data Collection, Preservation and Analysis |  |
| 3.1 | RR | Do guidelines exist for data preservation? |  |
| 3.2 | RR | Do guidelines exist for data collection (interviews, electronic data, photographs, videotapes, test plans, etc.)? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 3.4 | RR | Was data properly preserved for the team? |  |
| 3.5 | I-PTM | Was access to the site restricted to prevent inadvertent alteration of the data? |  |
| 3.6 | RR | Were photographs taken promptly, prior to alteration of the data? |  |
| 3.7 | RR | Were data needs driven by the analysis needs? |  |
| 3.8 | RR | Was a list of needed data developed and tracked? |  |
| 3.9 | RR | Were data collection efforts prioritized by fragility of the data? |  |
| 3.10 | RR I-PTM I-SRQM | Were appropriate physical, people, paper/electronic and position data examined by the team? |  |
| 3.11 | RR I-GE | Were interviews conducted using questions that were: - Open-ended - Neutral - Non-threatening - Not leading |  |
| 3.12 | RR | Did appropriate personnel perform interviews? |  |
| 3.13 | RR | Were physical data analyzed in a controlled manner, using test plans as appropriate? |  |
| 3.14 | RR | Was calibration data documented for any instruments used? |  |
| 3.15 | RR | Was a chain-of-custody documented for all physical, paper and electronic data? |  |
| 3.16 | RR | Were paper/electronic data identified by revision and inventoried? |  |
| 3.17 | RR | Were photos properly marked and annotated and subsequently cataloged? |  |
| 3.18 | RR | Were photos annotated to indicate relevant information (see specific photography guidelines in the Incident Investigation Toolkit)? |  |
| 3.19 | RR | Do photos show orientation and relative size? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 4 |  | Data Analysis Techniques |  |
| 4.1 | RR | Does the program identify specific techniques to determine the causes of incidents and to establish corrective actions to prevent recurrence? |  |
| 4.2 | I-PTM | Is there a structured approach to incident investigation and near-miss investigation? Are there specific investigation methods that are used? |  |
| 4.3 | RR | Are the methods consistently used when performing analyses? |  |
| 4.4 | RR | Is a system used to ensure that logic questions are asked during the analysis? |  |
| 4.5 | RR | Is logic used to ensure all required conditions and events are considered? |  |
| 4.6 | RR | Are multiple causes explicitly considered by the methodologies? |  |
| 4.7 | RR | Are the data specific to who, what, where, when, how? |  |
| 4.8 | RR | Is the origin of data identified to allow assessment of its level of certainty – guess, hearsay, opinion, logical conclusion, “fact”? |  |
| 4.9 | RR | Are multiple levels of causes identified? |  |
| 4.10 | RR | Is each level of cause specifically and logically connected to the next level of cause? |  |
| 4.11 | RR | Are data used to verify and refute hypotheses when using a fault tree approach? |  |
| 4.12 | RR | Are logic tests used in addition to the time-based prompts for time-based approaches? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 5 |  | Root Cause Identification Note: Some methods do not specifically identify root causes. They are just deeper causes in a continuum of causes |  |
| 5.1 | RR | Does each root cause identify: - Something that management can control (so a recommendation can be written to address the cause)? - A basic, underlying cause? |  |
| 5.2 | RR | Is each root cause associated with a causal factor or observation? |  |
| 5.3 | RR | Are root causes associated with management system issues? |  |
| 5.4 | RR | Are multiple root causes identified? |  |
| 5.5 | I-PTM | What is a root cause? Can you provide some examples? |  |
| 6 |  | Developing Recommendations |  |
| 6.1 | RR | Is each recommendation tied to a cause (causal factor and/or root cause)? |  |
| 6.2 | RR | Are recommendations developed to address all four levels of recommendations (as appropriate)? |  |
| 6.3 | RR | Are recommendations developed at the highest level practical in the recommendation hierarchy? |  |
| 6.4 | RR | Are recommendations assigned to specific individuals for implementation? Are specific completion dates assigned? |  |
| 6.5 | I-SRQM | After an incident is investigated, how are human factors deficiencies corrected at the point of the event? At other locations? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 7 |  | Completing the Investigation |  |
| 7.1 | RR | Reports should include key information - Date and time of the incident - Date and time the investigation began - Description of event - Causal factors (factors that contributed to the incident) - Underlying causes (root causes) - Recommendations |  |
| 7.2 | I-PTM | What items are addressed/included in the report? Are there specific items that should be excluded from the report? |  |
| 7.3 | RR | Are the analysis tools (CFC, 5-Whys and FTA) used to reduce the effort required to write the report? |  |
| 7.4 | RR | Are the report-writing guidelines and report checklist in the Incident Investigation Toolkit used? |  |
| 7.5 | RR | Is the effectiveness of selected recommendations assessed by monitoring a leading indicator? |  |
| 7.6 | RR | Is the investigation process periodically critiqued? |  |
| 7.7 | RR | Do analyses result in a signed-off report and recommendations? |  |
| 7.8 | I-PTM | What type of report is generated following an incident investigation? |  |
| 7.9 | RR I-SRQM | How long does it take for a typical investigation to be completed? |  |
| 7.10 | RR | Has a system been established to promptly address and resolve the incident investigation report findings and recommendations? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 7.11 | RR | Have resolutions and corrective actions from incident investigation reports been documented? Who is assigned the responsibility of maintaining these records? |  |
| 7.12 | I-PTM | Is documentation maintained regarding the resolution of incident investigation findings? If so, how, where? Can you find one for me? |  |
| 7.13 | RR I-SRQM | Are recommendations reviewed in accordance with the organization’s management of change process? Does the health and safety organization review the recommendations for potential impact on worker safety, facility safety and environmental impact? Are recommendations reviewed by the quality organization for potential impact on customer satisfaction? |  |
| 7.14 | RR | Are recommendations completed in accordance with the assigned completion dates? How many times are completion dates revised before a recommendation is completed? |  |
| 7.15 | I-GE | Are corrective actions completed in a timely manner following completion of analyses? |  |
| 7.16 | I-SRQM | Are there enforced time limits for completing recommendations from incident investigations? |  |
| 7.17 | RR | Are investigation records available to employees and safety, quality and/or reliability committee members? |  |
| 7.18 | RR | Have incident investigation reports been reviewed with all affected personnel whose job tasks are relevant to the investigation findings, including contract employees, where applicable? - Are there minutes of meetings or other evidence of communicating the results of incident investigation reports to affected personnel? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 7.19 | I-PTM | How are lessons learned disseminated throughout the company? Are there requirements? |  |
| 7.20 | I-EA | How do affected personnel find out about the results of incident investigations? |  |
| 7.21 | RR I-SRQM | Are incident investigation reports and recommendations communicated to the safety, health and quality organizations? |  |
| 7.22 | I-GE | How do you find out about the results of incident investigations that occur in your work area? |  |
| 7.23 | I-GE | Are the findings uncovered during incident investigations addressed in a timely manner? |  |
| 7.24 | I-PTM | Are incident investigation reports maintained for any specific period of time? If so, how long? |  |
| 7.25 | I-SRQM | Are reporting statistics highly publicized? |  |
| 8 |  | Selecting Problems for Analysis |  |
| 8.1 | RR | Are personnel encouraged to report problems and unsafe conditions? - Is there any fear of reprisal? - Is there a proactive management philosophy of not placing blame for incidents in order to ensure openness, cooperation and volunteering of information from employees? |  |
| 8.2 | I-GE | Were you ever involved in an accident or incident? If so: - Did you report it? Why or why not? - Was the incident investigated? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 8.3 | I-GE | Are you reluctant to seek medical treatment for a minor injury? Why? |  |
| 8.4 | I-GE | Are you reluctant to report problems you see? Why? |  |
| 8.5 | I-GE | Is there training on how to report? |  |
| 8.6 | I-GE | What major problems have you seen here? |  |
| 8.7 | I-GE | What near-miss incidents have you witnessed or heard about? |  |
| 8.8 | I-GE | Are loss events and near misses investigated? |  |
| 8.9 | I-GE | What is your definition of a near miss? |  |
| 8.10 | I-SRQM | How are personnel encouraged to report? |  |
| 8.11 | I-SRQM | Is there training on how to report? |  |
| 8.12 | I-SRQM | Who is accountable for incident investigation and near-miss reporting? |  |
| 8.13 | I-SRQM | What criteria and procedures exist for reporting and investigating accidents and near misses? |  |
| 8.14 | I-SRQM | Are they followed consistently? |  |
| 8.15 | RR | Does the organization have a documented method for selecting incidents for analysis (acute or chronic analysis)? |  |
| 8.16 | I-SRQM | How is it decided to undertake a detailed investigation? Is the selection method used? |  |
| 8.17 | RR | Have analyses of chronic problems been performed? |  |
| 8.18 | RR | Is the near miss to incident ratio high (greater than 10)? |  |
| 8.19 | I-SRQM | Are relevant statistics compiled from incident investigation reports? |  |
| 8.20 | I-SRQM | Are reporting levels appropriate? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 9 |  | Trending |  |
| 9.1 | RR | Are methods in place to periodically review performance data to identify chronic problems? Are the review methods structured? Are the reviews scheduled? Have the reviews been completed in accordance with the schedule? |  |
| 9.2 | RR | Does the company collect data related to each incident? |  |
| 9.3 | RR | Is a database established for recording and analyzing incidents to develop trend information? |  |
| 9.4 | I-SRQM | Are data on human errors collected and made available to managers? How is this information used? |  |
| 9.5 | I-SRQM | What formal measures of safety/reliability/quality performance are tracked? Have the data been used as the basis for any management decisions? |  |
| 9.6 | I-SRQM | What is the current safety/reliability/quality performance of the facility? |  |
| 9.7 | I-SRQM | Is this record better or worse than your historical one? |  |
| 9.8 | I-SRQM | Is this record better or worse than your goals? |  |
| 9.9 | RR | Does the organization have a documented method for selecting chronic incidents for analysis? |  |
| 9.10 | RR | Is the selection method used? |  |
| 9.11 | RR | Is someone assigned the responsibility for analyzing data for chronic incidents? |  |
| 9.12 | RR | Is chronic data analysis performed periodically? |  |
| 9.13 | RR | Have selected recommendations been selected for results assessment? |  |
| 9.14 | RR | Is someone assigned the responsibility of performing long-term results assessment? |  |
| 9.15 | RR | Are analyses being performed to assess the effectiveness of recommendations? |  |
| 9.16 | I-SRQM | Have the data from incident investigations been used as the basis for any management decisions? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 10 |  | Investigation Programs |  |
| 10.1 | RR | Does the incident investigation program address the interface with investigation of health, safety, reliability and quality problems? |  |
| 10.2 | RR | Does the organization have established training and experience requirements for investigation team leader(s) and members? Do the training requirements address both initial and ongoing training? |  |
| 10.3 | RR | Are training requirements established for all members of the organization (including those that do not perform incident investigations)? As a minimum, do these training requirements include the types of incidents to report and how to preserve evidence? Is ongoing training required for investigation team leaders? |  |
| 10.4 | RR | Does the organization have assigned responsibilities for reporting and investigating incidents, selecting team members, following-up and closing out of action items and training/communicating the results of incident investigations? |  |
| 10.5 | RR | Has the incident investigation program been revised recently? Are all the requirements current? |  |
| 10.6 | RR | Has each incident been investigated that exceeded the thresholds defined in the incident investigation program? |  |
| 10.7 | RR | Does the program meet all appropriate regulatory requirements? |  |
| 10.8 | RR I-PTM I-SRQM | Has management taken an active role in ensuring the program is functional? |  |
| 10.9 | I-PTM I-SRQM | What criteria does management use to evaluate an analysis? |  |

Incident Investigation/Root Cause Analysis Program Evaluation Checklist (continued)

| Item Number | Data collection method* | Incident Investigation Analysis Program Evaluation Checklist | Comments |
| --- | --- | --- | --- |
| 10.10 | I-PTM I-SRQM | Has management defined a goal for the investigation program? |  |
| 10.11 | I-PTM I-SRQM | Who is the organizational champion for the program? |  |
| 10.12 | I-PTM | What are the perceived rewards and punishments for participating or leading an investigation team? |  |
| 10.13 | RR | Has the role of the organization’s legal department in investigations been defined? |  |
| 10.14 | RR | Does the organization have someone assigned to address media requests related to an investigation? |  |
| 10.15 | RR | Are reports written in a neutral, factual tone to avoid media and legal problems? |  |
| 10.16 | I-GE | What do you think about the way incidents are investigated? |  |
| 10.17 | I-GE | Have you been involved in an accident or near miss? |  |
| 10.18 | RR | Are incident investigation reports retained until the next risk/reliability/safety assessment of the system? |  |
| 10.19 | RR | How many incident investigations were performed in the last 3 years? Is the number of investigations increasing or decreasing? |  |
| 10.20 | RR | Is management taking any actions to reduce the number of incidents? - If not, why not? |  |
| 10.21 | I-SRQM | Do the investigations go into enough depth to identify the root causes of human errors and other failures? |  |

RR – Records Review

FV – Field Verification

I-PTM – Interview Members Of Past Investigation Teams

I-EE – Interview Employees Affected By Past Investigation Results

I-GE – Interview General Employees

I-SRQM – Interview Safety, Reliability and Quality Managers

I-ERM – Interview Emergency Response Team Manager

This Page Intentionally Left Blank